SpiRob Physics Model Pipeline

Technical Reference & User Guide

# Overview

SpiRob is a cable-driven soft robot with a logarithmic spiral geometry. This pipeline converts a small set of physical parameters into a fully simulation-ready MuJoCo model in three automated steps:

- CSV generation — computes per-element geometry (joint positions, cable attachment sites) from the spiral equations

- STL generation — builds tapered 3-D meshes for every link using CadQuery

- XML generation — writes a MuJoCo MJCF file referencing the STL meshes, with correct tendon routing, joint properties, and actuators

# Requirements

Python 3.9+ with the following packages (see requirements.txt):

- cadquery  — solid geometry kernel (pinned git commit for OCC compatibility)

- numpy, pandas  — array math and CSV I/O

- matplotlib  — interactive preview window

- mujoco  — for loading and testing the generated XML

pip install -r requirements.txt

# Project Structure

| File | Purpose |
| --- | --- |
| build.py | Pipeline driver. Run this to generate everything. |
| params.json | All user-facing parameters — edit this to change the robot design. |
| spirob_csv_generator.py | Step 1 — spiral maths → per-element joint + tendon site CSV. |
| csv2geom_nlobe.py | Step 2 — CSV → STL meshes (flat / n-lobe / plain revolve). |
| csv2xml.py | Step 3 — CSV + STL → MuJoCo MJCF XML. |
| preview.py | Interactive 2-D geometry preview with hover/click inspection. |
| helper_functions.py | Shared spiral maths (solve_b, generate_spiral_pose, etc.). |

Generated outputs (not committed to git): Geom_Data_CSV/,  meshes/,  spirob_physics_model.xml.

# Quick Start

## 1. Edit params.json

Set your physical dimensions and cross-section choice (see Parameters Reference below), then run:

## 2. Run the pipeline

python build.py --nlobe

This will:

- Validate all parameters and abort with a clear message if anything is wrong

- Show the interactive 2-D side-view preview — inspect the geometry, then type y to proceed

- Show the cross-section preview window (base and tip elements side by side)

- Generate CSV → STL → XML in sequence

## 3. Load in MuJoCo

python -m mujoco.viewer --mjcf=spirob_physics_model.xml

# Build Flags

| Flag | Effect |
| --- | --- |
| --nlobe | n-lobe or flat cross-section driven by n_cables. Use this for all real builds. |
| --plain | Full solid of revolution — no cross-section cut. Useful for debugging. |
| --safe | MuJoCo safe-mode preset: capsule collision, high damping, small timestep. |
| --fast | Lightweight preset: mesh physics, low damping, large timestep. |
| --high | High-fidelity preset: RK4 integrator, fine timestep, tight margins. |
| --noclean | Keep previous CSV/STL/XML outputs (skip the clean step). |
| --no-preview | Skip interactive preview — useful for batch / CI runs. |
| --params FILE | Use a different params JSON file (default: params.json). |

Flags can be combined. Examples:

python build.py --nlobe --safe --no-preview

python build.py --nlobe --high --params params_v2.json

# Parameters Reference  (params.json)

## Core Geometry

| Parameter | Type | Valid range | Description |
| --- | --- | --- | --- |
| L | float (m) | > 0 | Total uncoiled length of the robot. |
| d_tip | float (m) | > 0 | Outer diameter at the tip (smallest end). |
| phi_deg | float (°) | (0, 45) | Taper half-angle of the logarithmic spiral. Controls how quickly the robot widens from tip to base. |
| Delta_theta_deg | float (°) | (0, 180) | Angular span of each element along the spiral. Smaller values = more elements and finer resolution. |
| n_cables | int | ≥ 2 | Number of actuation cables. Also selects the cross-section: 2 → flat slab, 3+ → n-lobe polygon. |
| tendon_inward_shift | float (m) | [0, d_tip/2) | How far to pull cable attachment sites inward from the outer surface. |
| show_preview | bool | true/false | Whether to launch the interactive preview when running preview.py standalone. |

## Cross-Section  (n_cables >= 3  —  n-lobe)

| Parameter | Type | Valid range | Description |
| --- | --- | --- | --- |
| nlobe_t | float | [0, 1] | 0 = circumscribed polygon (vertices touch outer wall, no arcs visible). 1 = inscribed polygon (flat edges tangent, maximum arc exposure). 0.5 = balanced default. |
| notch_factor | float | [0, 0.4] | Notch circle radius as a fraction of the polygon half-side length. Notches are cut at each edge midpoint for mass and inertia reduction. |

## Cross-Section  (n_cables <= 2  —  flat slab)

| Parameter | Type | Valid range | Description |
| --- | --- | --- | --- |
| flat_thickness_ratio | float | [0.05, 1.0] | Element thickness as a fraction of its outer radius. Drives the Y-extrusion depth of the trapezoidal slab. |

## post_gen  —  post-generation overrides

All fields in the post_gen block are optional. Omit any field to keep the pipeline default. These values are applied by csv2xml.py during XML generation — no separate script needed.

| Field | Type | Description |
| --- | --- | --- |
| robot_pos | [x, y, z] | World-frame position of the base link (link_001). Default: computed from CSV. |
| robot_quat | [w, x, y, z] | World-frame orientation of the base link as a unit quaternion. |
| joint_stiffness_base | float | Stiffness of joint 0 (base). All subsequent joints decay geometrically: k_i = base / β³ⁱ. |
| joint_damping_base | float | Damping of joint 0. Same β³ decay applies. |
| joint_beta | float > 0 | Decay rate β. β > 1 = decreasing stiffness/damping toward the tip. β = 1 = uniform. Default: 1.03. |
| first_joint_stiffness | float | Independent override for joint 0 stiffness only — applied after the β decay chain. |
| first_joint_damping | float | Independent override for joint 0 damping only. |
| tip_site_pos | [x, y, z] | Position of a green sphere site attached to the last (tip) body. Useful for tracking end-effector position. |
| target_site_pos | [x, y, z] | Position of a red sphere site placed in the world frame. Useful as a visual target. |

# Cross-Section Modes

csv2geom_nlobe.py is the single STL generator for all modes. The mode is determined automatically from params.json and the --plain flag:

| Condition | Joint type | Description |
| --- | --- | --- |
| --plain flag set | ball | Full solid of revolution. No cross-section cut. Used for debugging or simple visual inspection. |
| n_cables <= 2 | hinge | Flat tapered slab extruded from the actual trapezoidal quad profile in ±Y. Mimics a compliant hinge mechanism. |
| n_cables >= 3 | ball | Revolved cylinder intersected with a regular n-gon (n = n_cables sides), with circular notches at each edge midpoint. A trilobe is simply n_cables = 3. |

The hinge / ball joint type is determined automatically by build.py — no manual flag needed.

# Geometry Preview

The preview gate runs before any geometry is generated, letting you verify parameters without waiting for STL output.

## Side-view preview (always shown)

- All elements drawn as filled quads with an opacity gradient (dark = base, light = tip)

- Total length arrow, tip/base radius arrows, taper angle arc

- Tendon path (cable 0) shown as a dashed red line — toggle with the checkbox

- Hover over any element to inspect its height, top/bottom width, and Z position

- Click an element to pin the selection (orange highlight)

## Cross-section preview (shown with --nlobe)

- Side-by-side panels: base element (largest) and tip element (smallest)

- n_cables <= 2: actual trapezoidal profile with width and thickness annotations

- n_cables >= 3: polygon with notch circles, outer radius and notch radius annotated

## Standalone usage

python preview.py --params params.json

python preview.py --params params.json --nlobe   # also show cross-section

python preview.py --params params.json --proceed  # skip yes/no, exit 0

Exit codes: 0 = approved, 1 = rejected or window closed. build.py reads these exit codes to gate the pipeline.

# MuJoCo Physics Presets

Pass one preset flag to build.py. If no preset is given, the default MJCFConfig values are used.

| Preset | Timestep | Integrator | Best for |
| --- | --- | --- | --- |
| --safe | 0.001 s | implicit | Initial bring-up. Capsule collision, hinge range ±15°, high damping. |
| --fast | 0.005 s | implicit | Fast iteration. Mesh physics, wide control range, minimal damping. |
| --high | 0.0005 s | RK4 | Accurate dynamics. Fine timestep, tight geometry margins. |
| (default) | 0.002 s | implicit | Balanced. Mesh physics, moderate damping and stiffness. |

# Joint Stiffness & Damping Decay

Real soft robots are stiffer at the base and more compliant at the tip. The pipeline models this with a geometric decay:

k_i  =  k_base / β³ⁱ      d_i  =  d_base / β³ⁱ

where i = 0 is the base joint and β is joint_beta in post_gen. With β = 1.03 (the default), joint 20 has roughly 17× less stiffness than joint 0.

Use first_joint_stiffness and first_joint_damping to independently override joint 0 — useful when the base attachment is mechanically much stiffer than the first flexible joint.

# Git Workflow

Generated outputs are large, change every run, and are always fully reproducible from params.json alone. They must not be committed.

## Files to commit

- build.py

- spirob_csv_generator.py

- csv2geom_nlobe.py

- csv2xml.py

- preview.py

- helper_functions.py

- params.json

- requirements.txt

- README.md

- .gitignore

## Files excluded by .gitignore

- Geom_Data_CSV/  — generated CSV

- meshes/  — generated STL files

- spirob_physics_model.xml  — generated XML

- __pycache__/  — Python bytecode

- *.pyc

# .gitignore

# Generated outputs — always reproducible from params.json

Geom_Data_CSV/

meshes/

spirob_physics_model.xml

# Python

__pycache__/

*.pyc

*.pyo

.env

venv/

# Troubleshooting

Pipeline aborts with a parameter error

Read the error message — validate_params() lists every problem. Fix params.json and re-run.

STL generation fails on some elements with 'BRepAdaptor_Curve::No geometry'

This is an OCC topology error caused by degenerate geometry. Common causes: Delta_theta_deg too large (try reducing by ~20%), or nlobe_t at exactly 0 or 1 with extreme notch_factor. Use the preview to check element proportions first.

Bodies all appear at the same location in MuJoCo

Check that robot_pos and robot_quat in post_gen are set correctly. The default robot_pos [0, 0, 0] places the base at the world origin — the robot may be clipping through the ground plane. Set robot_pos to [0, 0, L] where L is your robot length.

Tendons appear to miss the link geometry

Increase tendon_inward_shift slightly. The preview shows the corrected tendon path as a dashed red line — use it to visually verify placement before generating STL.

Simulation is unstable / exploding

Start with --safe mode. If stable, switch to --fast, then the default. If still unstable, reduce timestep in post_gen or increase joint_damping_base.